# UCM (backpack: GPS, microclimate, radiation, wind, acoustics, air quality)

ucm__gps_lat — Geographic latitude of the wearer’s position. Helps map exposure by place.

Unit: degree (°); 1° ≈ 111 km north–south.

Bounds: 49–52.  Typical: U/G ~50.80–50.92.

ucm__gps_lon — Geographic longitude of the wearer’s position.

Unit: degree (°); east–west spacing shrinks with latitude.

Bounds: 2–7.  Typical: U/G ~4.25–4.45.

ucm__gps_alt — GPS altitude above mean sea level; rough terrain/context cue.

Unit: metre (m).

Bounds: −10–500.  Typical: U 10–70; G 10–130.

ucm__gps_hdop — Horizontal dilution of precision; lower = better GPS geometry (tall buildings worsen it).

Unit: unitless.

Bounds: 0–50.  Typical: U 1–5; G 0.5–2.

ucm__gps_speed — Ground speed; distinguishes walk/bike/tram and affects true wind calc.

Unit: m/s (1 m/s = 3.6 km/h).

Bounds: 0–50.  Typical: Walk 0–2; Bike 2–8; Tram 0–20 (peaks).

ucm__gps_hdg — GPS course/heading; direction of travel relative to north.

Unit: degree (°).

Bounds: 0–360 (wrapped).  Typical: U/G any (route dependent).

ucm__mag_hdg — Magnetic heading from magnetometer; wearer’s facing orientation.

Unit: degree (°).

Bounds: 0–360 (wrapped).  Typical: U/G any (posture dependent).

ucm__air_temp — Ambient air temperature; key for thermal load.

Unit: degree Celsius (°C).

Bounds: 10–60.  Typical: U 24–34; G 23–32.

ucm__air_rh — Relative humidity; moisture share of air, shapes heat stress.

Unit: percent (%).

Bounds: 0–100.  Typical: U 35–65; G 40–70.

ucm__air_t_bot / __mid / __top — Air temperature at multiple heights; vertical gradient proxy.

Unit: °C.

Bounds: 0–80.  Typical: U 24–35; G 23–33 (small gradients).

ucm__ir_back / __front / __left / __right / __up / __down — Directional IR “brightness” temps; apparent surface temps in each direction (sunlit surfaces read hot).

Unit: °C.

Bounds: 0–80.  Typical: Sunlit facades 35–60; shade 20–35; canopy shade 18–35.

ucm__ir_spot_gnd / __left / __right / __sky — Narrow field IR spot temps of ground/sides/sky.

Unit: °C.

Bounds: 0–80.  Typical: Ground sun 35–60; shade 20–35; “sky” low (often 0–25 in your non-negative export).

ucm__sun_gh — Global horizontal irradiance; total solar power on a horizontal plane.

Unit: watt per square metre (W/m²).

Bounds: 0–1400.  Typical: U open sun 400–900; G under canopy 50–400.

ucm__sun_alt — Solar altitude; sun height above horizon (time-of-day/season cue).

Unit: degree (°).

Bounds: −10–90.  Typical: Jul–Aug afternoon ~25–55.

ucm__sun_az — Solar azimuth; sun’s compass bearing (north=0°).

Unit: degree (°).

Bounds: 0–360 (wrapped).  Typical: Afternoon ~200–290 (SW→WNW).

ucm__snd_dba — A-weighted sound pressure level; human-hearing weighted loudness.

Unit: dBA (decibels A-weighted).

Bounds: 20–140.  Typical: U 55–85; G 45–70.

ucm__aq_pm010 (PM1), __aq_pm025 (PM2.5), __aq_pm100 (PM10) — Particulate matter mass concentrations; finer → deeper lung penetration.

Unit: microgram per cubic metre (µg/m³).

Bounds: 0–1000 (QC; extreme dust events aside).  Typical: PM2.5 U 8–40 / G 4–25; PM10 U 10–80 / G 8–50 (PM1 lower).

ucm__aq_co2 — Carbon dioxide; outdoor background + local accumulation in canyons/crowds.

Unit: parts per million (ppm).

Bounds: 300–20,000.  Typical: U 450–700; G 420–550.

ucm__aq_no2_m1 — Nitrogen dioxide; traffic exhaust tracer.

Unit: parts per billion (ppb).

Bounds: 0–5000.  Typical: U 10–80; G 5–40.

ucm__aq_o3_m1 — Ozone; regional photochemical pollutant (often higher away from fresh NO).

Unit: ppb.

Bounds: 0–5000.  Typical: U 20–80; G 30–100.

ucm__aq_so2_m4 — Sulfur dioxide; combustion marker, usually low in Brussels.

Unit: ppb.

Bounds: 0–5000.  Typical: U/G 0–10 (near DL).

ucm__aq_co_m1 — Carbon monoxide; incomplete combustion marker.

Unit: ppm.

Bounds: 0–5000.  Typical: U 0–2; G 0–1.

ucm__wind_aws — Apparent wind speed (what the moving wearer feels); includes self-motion.

Unit: m/s.

Bounds: 0–50.  Typical: U 0–10; G open 0–12 (higher while biking).

ucm__wind_awa — Apparent wind angle; wind direction relative to wearer motion.

Unit: degree (°).

Bounds: 0–360 (wrapped).  Typical: U/G any.

ucm__wind_tws — True wind speed; ambient wind after correcting for motion.

Unit: m/s.

Bounds: 0–50.  Typical: U 0–6; G open 0–8.

ucm__wind_twd — True wind direction; compass bearing of ambient wind.

Unit: degree (°).

Bounds: 0–360 (wrapped).  Typical: U/G any (synoptic/local breeze dependent).

ucm__io_flag — Indoor/outdoor classifier; 1=indoor, 0=outdoor.

Unit: binary (0/1).

Bounds: 0–1.  Typical: Mostly 0 (outdoor campaign).

ucm__ah — Absolute humidity; actual water vapour mass per air volume.

Unit: gram per cubic metre (g/m³).

Bounds: 0–40.  Typical: U 8–15; G 9–17.

ucm__mrt — Mean radiant temperature; combined radiant load from all directions.

Unit: °C.

Bounds: 0–80.  Typical: U 30–65; G 25–50.

ucm__mrt_s — Short-wave radiant load proxy (instrument-specific scale).

Unit: often W/m² or °C-equiv (confirm device).

Bounds: 0–1400.  Typical: U 100–900; G 0–600.

ucm__opt — Operative temperature; combined air/radiant effect felt by a person.

Unit: °C.

Bounds: 0–60.  Typical: U 24–38; G 22–35.

ucm__humidex / __humidex_sr — Humidex; heat stress index from T and RH.

Unit: °C-equivalent.

Bounds: 0–80.  Typical: U 27–40; G 26–38.

ucm__utci / __utci_sr — Universal Thermal Climate Index; thermo-physiological heat/cold stress.

Unit: °C-equivalent.

Bounds: 0–80 (study QC).  Typical: U 26–38; G 24–36.

ucm__pet / __pet_sr — Physiological Equivalent Temperature; heat stress index.

Unit: °C-equivalent.

Bounds: 0–80.  Typical: U 24–40; G 22–38.

ucm__fp — Study-specific field/proximity score (contextual index you defined).

Unit: unitless.

Bounds: 0–4000 (kept wide without device spec).  Typical: Use dataset percentiles to interpret.

UCM other information –

1. Citation : "Monitoring data provided by using the microclimate-meter uCM LECEA - HEPIA"

2. 'sr' stands for stress range; these are defined as follows :

def HUMIDEX_SR(array):

# HUMIDEX StressRange

HUMIDEX_SR = []

for rows in range(len(array)):

HUMIDEX = array[rows]["HUMIDEX"]

if math.isnan(HUMIDEX) or HUMIDEX < 20:

HUMIDEX_SR.append(float('nan'))

elif HUMIDEX >= 20 and HUMIDEX < 30:

HUMIDEX_SR.append(float(0))

elif HUMIDEX >= 30 and HUMIDEX < 40:

HUMIDEX_SR.append(float(1))

elif HUMIDEX >= 40 and HUMIDEX < 45:

HUMIDEX_SR.append(float(2))

elif HUMIDEX >= 45:

HUMIDEX_SR.append(float(3))

else:

HUMIDEX_SR.append(float('nan'))

return HUMIDEX_SR

def UTCI_SR(array):

# UTCI StressRange

UTCI_SR = []

for rows in range(len(array)):

utci = array[rows]['UTCI']

if math.isnan(utci):

UTCI_SR.append(float('nan'))

elif utci >= 46:

UTCI_SR.append(float(4))

elif utci >= 38 and utci < 46:

UTCI_SR.append(float(3))

elif utci >= 32 and utci < 38:

UTCI_SR.append(float(2))

elif utci >= 26 and utci < 32:

UTCI_SR.append(float(1))

elif utci >= 9 and utci < 26:

UTCI_SR.append(float(0))

elif utci >= 0 and utci < 9:

UTCI_SR.append(float(-1))

elif utci >= -13 and utci < 0:

UTCI_SR.append(float(-2))

elif utci >= -27 and utci < -13:

UTCI_SR.append(float(-3))

elif utci >= -40 and utci < -27:

UTCI_SR.append(float(-4))

else:

UTCI_SR.append(float(-5))

return UTCI_SR

def PET_SR(array):

# PET StressRange

PET_SR = []

for rows in range(len(array)):

pet = array[rows]["PET"]

if math.isnan(pet):

PET_SR.append(float('nan'))

elif pet > 41:

PET_SR.append(float(4))

elif pet > 35 and pet <= 41:

PET_SR.append(float(3))

elif pet > 29 and pet <= 35:

PET_SR.append(float(2))

elif pet > 23 and pet <= 29:

PET_SR.append(float(1))

elif pet >= 18 and pet <= 23:

PET_SR.append(float(0))

elif pet >= 13 and pet < 18:

PET_SR.append(float(-1))

elif pet >= 8 and pet < 13:

PET_SR.append(float(-2))

elif pet >= 4 and pet < 8:

PET_SR.append(float(-3))

else:

PET_SR.append(float(-4))

return PET_SR

# Empatica (physiology & activity)

empatica__pulse_rate_bpm — Heart rate; cardiovascular response to effort/heat/stress.

Unit: beats per minute (bpm).

Bounds: 30–240.  Typical: U 70–160; G 65–150 (by activity).

empatica__respiratory_rate_brpm — Breathing rate; rises with exertion/heat.

Unit: breaths per minute (brpm).

Bounds: 0–60.  Typical: U/G 10–30.

empatica__eda_scl_usiemens — Skin conductance level; sympathetic arousal/thermal sweating.

Unit: microsiemens (µS).

Bounds: 0–50.  Typical: U 0.5–8 (spikes to ~15–20); G 0.3–6.

empatica__temperature_celsius — Wrist/skin temperature; slower-moving than air T.

Unit: °C.

Bounds: 10–60.  Typical: U/G 28–36.

empatica__accelerometers_std_g — Acceleration variability; movement intensity proxy.

Unit: g (1 g ≈ 9.81 m/s²).

Bounds: 0–8.  Typical: Tram low; Walk moderate; Bike high (0–2+).

empatica__counts_[x|y|z]_axis / _acticounts[x|y|z]_axis / __activity_counts / __vector_magnitude — Activity metrics from raw accelerometry.

Unit: unitless (device scales).

Bounds: ≥0 (no hard upper bound).  Typical: Low tram, medium walk, high biking; interpret via percentiles.

empatica__met — Metabolic equivalent; energy cost estimate.

Unit: MET (1 MET ≈ 3.5 ml O₂/kg/min).

Bounds: 0–20.  Typical: Tram 1–2; Walk 2–4; Bike 4–10.

empatica__prv_rmssd_ms — HRV (RMSSD); higher at rest, lower under stress/exertion.

Unit: millisecond (ms).

Bounds: 0–500.  Typical: U/G 15–100 (drops with effort/heat).

empatica__step_counts — Steps detected in the minute.

Unit: count.

Bounds: 0–500.  Typical: Walk 80–130; Bike/Tram ~0 (occasional false steps).

empatica__sleep_detection_stage — Sleep stage classifier (daytime noise possible).

Unit: categorical 0–4.

Bounds: 0–4.  Typical: 0 (awake) during your sessions.

empatica__wearing_detection_percentage — Confidence the device is worn.

Unit: percent (%).

Bounds: 0–100.  Typical: 80–100 when on; 0 when off.

# LYS button (personal light & color) — applies to lys1__* and lys2__*

lys*__lux — Illuminance; total visible light at chest level.

Unit: lux (lumens per square metre).

Bounds: 0–120,000.  Typical: U sun 30k–90k; U shade 200–10k; G canopy 50–5k; sun patches 20k–80k.

lys*__kelvin — Correlated color temperature; “coolness/warmth” of light spectrum.

Unit: kelvin (K).

Bounds: 1,000–25,000.  Typical: Outdoor daylight ~4,500–9,000 (higher in shade/overcast).

lys*__rgbr / __rgbg / __rgbb / __rgbir — Raw channel intensities incl. IR; spectral composition proxy.

Unit: unitless counts.

Bounds: ≥0 (no strict upper cap; device-scale).  Typical: Rise with brightness; compare within-device percentiles.

lys*__r' / __g' / __b' — Normalized chromatic components (0–1); color proportions.

Unit: fraction (0–1).

Bounds: 0–1.  Typical: Daylight: g′ often dominant; b′ higher in shade/canopy.

lys*__medi — Melanopic equivalent daylight illuminance; light effective for circadian/alerting.

Unit: melanopic lux.

Bounds: 0–100,000 (practical).  Typical: U sun 5k–40k; U shade 100–2k; G canopy 30–1.5k.

lys*__movement — Device motion index; helps identify activity artifacts.

Unit: unitless.

Bounds: ≥0 (no strict upper cap).  Typical: Tram low; Walk medium; Bike higher (relative scale).

# Atmotube Pro (portable AQ) — applies to atmotube_left__* and atmotube_right__*

atmotube_*__voc — Volatile organic compounds; mix of gaseous organics near sources.

Unit: ppm (H₂-equivalent, device index).

Bounds: 0–10.  Typical: U 0.1–0.8; G 0.05–0.5 (transient spikes near traffic/solvents).

Atmotube actually measures TVOC (a broad, MOX-based “equivalent VOC” in ppb).

“VOC” = all volatile organic compounds. There are two big sources:

- Biogenic VOCs (BVOCs) from vegetation (isoprene, monoterpenes). These can be higher in green areas, especially on warm, sunny summer afternoons and near high-emitting species (e.g., oaks, some conifers).

- Anthropogenic VOCs from traffic, fuels, solvents, cooking, etc. These are typically higher in urban corridors and around roads, workshops, and restaurants. (Source in Appendix)

atmotube_*__aqs — Air Quality Score; higher is cleaner per device algorithm.

Unit: unitless (0–100).

Bounds: 0–100.  Typical: U 40–90; G 60–100.

atmotube_*__temperature — Ambient temperature near the wearer.

Unit: °C.

Bounds: 10–60.  Typical: U 24–34; G 23–32.

atmotube_*__humidity — Relative humidity near the wearer.

Unit: %.

Bounds: 0–100.  Typical: U 35–65; G 40–70.

atmotube_*__pressure — Barometric pressure; weather/altitude cue.

Unit: millibar (mbar ≈ hPa).

Bounds: 850–1100.  Typical: U/G 990–1030.

atmotube_*__pm1 / __pm2.5 / __pm10 — Particulate matter mass; traffic and dust exposure.

Unit: µg/m³.

Bounds: 0–1000.  Typical: PM2.5 U 8–40 / G 4–25; PM10 U 10–80 / G 8–50; PM1 lower.

Atmotube PRO 2 Technical Specifications | ATMO Support Center

# Eyetracker (Pupil Labs Neon)

eyetracker__pupil_diameter_left / __right / __avg — Pupil size; contracts in bright light, dilates in low light/cognitive load.

Unit: millimetre (mm).

Bounds: 1–9.  Typical: U sun 2–3; U shade 3–4.5; G 3–5.

eyetracker__eyelid_aperture_left / __right / __avg — Eyelid opening; drops during blinks/squinting (glare, wind).

Unit: unitless index or mm (pipeline-specific).

Bounds: ≥0 (pipeline scale).  Typical: Participant-relative; lower during glare/fatigue.

eyetracker__fixation_count / __fixation_rate — Number/rate of fixations; visual sampling of the scene.

Unit: count per minute (and count).

Bounds: ≥0.  Typical: 60–250/min; often shorter and fewer during high motion.

eyetracker__fixation_duration_scaled — Normalized fixation duration; higher means longer looks.

Unit: fraction (0–1).

Bounds: 0–1.  Typical: 0.2–0.7 (task/scene dependent).

eyetracker__gaze_dispersion (if present) — Spread of gaze points; scene scanning vs focused viewing.

Unit: degree visual angle (°) or pixel.

Bounds: ≥0.  Typical: 1–10°; wider during navigation.

eyetracker__gaze_velocity (if present) — Speed of gaze motion; captures saccades/smooth pursuit.

Unit: degree per second (°/s).

Bounds: ≥0.  Typical: 0–400°/s with saccadic peaks.

eyetracker__stress_composite — Ocular stress proxy from multiple eye features.

Unit: fraction (0–1).

Bounds: 0–1.  Typical: U 0.2–0.6; G 0.1–0.5.

eyetracker__audio_speech_probability — Likelihood of speech in the audio channel.

Unit: fraction (0–1).

Bounds: 0–1.  Typical: U 0–0.8; G 0–0.4.

eyetracker__audio_stress_level — Audio-derived stress/noise proxy.

Unit: fraction (0–1).

Bounds: 0–1.  Typical: U 0.1–0.7; G 0–0.5.

Basic Eye Tracking Measures

- gaze x [px], gaze y [px]: Screen coordinates where the person is looking

- worn: Whether eye tracking device was being worn

- gaze_velocity: How quickly the eyes are moving (pixels per second)

Eye Events

- in_fixation: Whether eyes are fixed on one point

- fixation_duration_s: How long each fixation lasts (seconds)

- in_saccade: Whether eyes are making quick movements between fixations

- saccade_amplitude: How large the eye movement is (degrees)

- saccade_peak_velocity: Maximum speed during eye movement

- in_blink: Whether eyes are blinking

- blink_duration_s: How long each blink lasts (seconds)

Pupil and Eyelid Measures

- pupil_diameter_left/right/avg: Pupil size (mm)

- pupil_change_rate: How quickly pupil size is changing

- eyelid_aperture_left/right/avg: How open the eyes are (mm)

Gaze Location Metrics

- distance_from_center: How far gaze is from screen center (pixels)

- gaze_centrality: How close to center (0=edge, 1=center)

- gaze_quadrant: Which screen quadrant (top-right, top-left, etc.)

- gaze_dispersion: How scattered recent gaze points are

Head Movement (IMU data)

- gyro_x, gyro_y, gyro_z: Head rotation (degrees/second)

- accel_x, accel_y, accel_z: Head acceleration (g)

Event Counts

- fixation_count: Number of fixations in 10-second window

- fixation_rate: Fixations per second

Stress Metrics

- fixation_count_scaled: Normalized fixation count (higher=more stress)

- fixation_duration_scaled: Normalized fixation duration (higher=more stress)

- stress_composite: Overall stress score (0-1 scale)

- stress_level: Categorical stress level (Low, Moderate, High, Very High)

What These Variables Mean

1. pupil_diameter_avg

This is the average size of your pupil. When people are stressed, focused, or in bright/dark environments, their pupil size changes.

2. pupil_change_rate

This tells us how fast the pupil size is changing. Quick changes can mean sudden reactions or changes in light or emotions.

3. eyelid_aperture_avg

This is how open your eyes are on average. Smaller openings can mean you're tired, bored, or stressed. Wider eyes can mean you're alert or surprised.

4. fixation_duration_s

This shows how long your eyes stay focused on one spot. Longer fixations often mean you're thinking hard or really interested in something.

5. fixation_count

This counts how many times your eyes stop to look at things. More fixations can mean you're exploring or trying to understand something.

6. saccade_amplitude

This shows how far your eyes jump between fixations. Bigger jumps often happen when you're scanning a lot or looking for something specific.

7. audio_rms

This measures how loud the environment is. Louder sounds can add to stress or distraction.

8. stress_composite

This is a score that combines many body signals and sounds to estimate how stressed you might be overall.

9. audio_stress_indicator

This looks at sounds and speech around you to guess if the environment might be stressful or tense.