# Data Processing Methodology

# 1. Overview

This document describes the end-to-end data processing pipeline for the field experiment on urban mobility and environmental comfort. Seventeen participants (P2–P18) wore or carried five different sensor systems across five experiment phases per session: BikeU (urban cycling), WalkU (urban walking), BikeG (green cycling), WalkG (green walking), and Tram (seated transit). All outputs are synchronised to a common 10-second time backbone in UTC and stored in the outputs/ folder.

The pipeline is implemented as a numbered sequence of Python scripts. Each script is independent (reads raw data or previous outputs only) and must be run in order. A shared key file (key.csv) defines exact UTC start/end times for every participant × phase combination and drives phase-window filtering in all scripts.

# 2. Folder Structure

Data Processing_05May/

├── key.csv                       ← Phase windows (UTC start/end per participant × phase)

├── scripts/                      ← All processing scripts

│   ├── 00_build_index.py

│   ├── 01_build_empatica.py      ← NeuroKit2-enhanced Empatica reprocessing

│   ├── 02_build_ucm.py

│   ├── 03_build_atmo_lys.py

│   ├── 04_build_eyetracker.py

│   ├── 05_merge_all.py

│   └── empatica_quality_check.py

├── outputs/

│   ├── index_10sec.csv           ← Time backbone

│   ├── empatica_corrected_10sec.csv

│   ├── ucm_10sec.csv

│   ├── atmo_lys_merged.csv

│   ├── eyetracker_10sec.csv

│   └── (quality report .html files)

└── raw data/

├── Empatica Data/

├── Atmo & LYS/rawdata/

├── ucm/ (via Participantwise folder)

└── Eyetracker/Processed eyetracker/

# 3. Key File — key.csv

key.csv is the single source of truth for all phase boundaries. It contains one row per participant and columns for the UTC start and end of each phase:

Columns: Participant_ID, Name, Date (DD-Mon format), BikeU_start, BikeU_end, WalkU_start, WalkU_end, BikeG_start, BikeG_end, WalkG_start, WalkG_end, Tram_start, Tram_end

All times are stored in UTC.

Sensor files (LYS, Atmotube) record in Brussels local time (CEST = UTC+2). All build scripts apply the +2h offset when filtering sensor data and subtract it again before joining the UTC index.

| PID | Date | BikeU_start | BikeU_end | WalkU_start | WalkU_end | BikeG_start | BikeG_end |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 11-Jul | 14:01:56 | 14:26:26 | 13:17:41 | 13:41:11 | 14:40:56 | 14:57:26 |
| 3 | 12-Jul | 12:31:42 | 12:55:25 | 11:56:40 | 12:23:16 | 13:21:25 | 13:35:55 |
| 4 | 25-Aug | 12:41:40 | 13:02:59 | 13:10:13 | 13:42:51 | 11:58:44 | 12:11:29 |
| 5 | 21-Jul | 14:42:25 | 15:04:10 | 15:17:10 | 15:48:10 | 13:32:56 | 13:49:11 |
| 6 | 28-Jul | 12:37:58 | 13:01:28 | 13:09:49 | 13:37:34 | 14:04:57 | 14:24:12 |
| 7 | 01-Aug | 13:36:08 | 14:08:53 | 14:14:56 | 14:44:11 | 12:15:12 | 12:32:27 |
| 8 | 03-Aug | 11:33:07 | 12:04:07 | 12:16:22 | 12:44:07 | 13:44:27 | 13:59:42 |
| 9 | 06-Aug | 10:16:32 | 10:39:02 | 10:43:58 | 11:16:43 | 11:46:57 | 12:01:11 |
| 10 | 12-Aug | 11:47:19 | 12:12:05 | 12:17:12 | 12:45:27 | 13:15:13 | 13:29:19 |
| 11 | 09-Aug | 13:21:17 | 13:40:32 | 12:43:50 | 13:12:50 | 12:03:46 | 12:22:55 |
| 12 | 10-Aug | 12:49:34 | 13:08:19 | 13:13:14 | 13:40:30 | 12:10:11 | 12:24:08 |
| 13 | 13-Aug | 12:49:14 | 13:10:14 | 13:17:44 | 13:46:59 | 12:11:02 | 12:23:02 |
| 14 | 14-Aug | 12:00:35 | 12:19:20 | 12:24:05 | 12:56:50 | 13:32:55 | 13:47:25 |
| 15 | 15-Aug | 13:31:33 | 13:49:33 | 12:55:05 | 13:26:20 | 12:11:02 | 12:25:32 |
| 16 | 17-Aug | 13:29:45 | 13:47:00 | 13:49:49 | 14:17:05 | 12:44:02 | 12:58:02 |
| 17 | 26-Aug | 13:11:01 | 13:32:31 | 12:36:17 | 13:07:32 | 11:51:03 | 12:04:35 |
| 18 | 27-Aug | 11:41:08 | 11:56:38 | 12:00:56 | 12:25:11 | 12:57:50 | 13:12:32 |

| PID | WalkG_start | WalkG_end | Tram_start | Tram_end | Indoor_start | Indoor_end |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 15:18:56 | 15:33:26 | 16:04:11 | 16:13:41 | 13:41:26 | 13:59:11 |
| 3 | 13:04:55 | 13:20:10 | 13:42:55 | 13:52:55 | 12:23:25 | 12:28:49 |
| 4 | 11:39:34 | 11:56:26 | 12:21:44 | 12:32:59 | 12:35:44 | 12:37:29 |
| 5 | 13:54:11 | 14:10:11 | 16:16:57 | 16:27:27 | 14:21:26 | 14:28:55 |
| 6 | 14:46:53 | 14:58:53 | 15:09:57 | 15:20:27 | 13:01:43 | 13:09:34 |
| 7 | 12:51:53 | 13:09:23 | 14:53:43 | 15:04:31 | 13:26:08 | 13:34:53 |
| 8 | 13:11:04 | 13:29:49 | 14:14:56 | 14:22:56 | 12:44:22 | 13:00:04 |
| 9 | 11:32:46 | 11:46:43 | 12:08:26 | 12:16:56 | 10:39:17 | 10:43:28 |
| 10 | 12:56:51 | 13:14:13 | 13:32:04 | 13:42:04 | 12:45:42 | 12:49:21 |
| 11 | 11:42:46 | 11:58:01 | 12:25:10 | 12:34:40 | 13:40:47 | 13:41:32 |
| 12 | 11:52:40 | 12:09:41 | 12:34:53 | 12:43:38 | 12:45:38 | 12:47:34 |
| 13 | 11:54:50 | 12:10:20 | 12:33:55 | 12:41:10 | 13:10:29 | 13:16:59 |
| 14 | 13:14:49 | 13:30:19 | 13:55:08 | 14:03:23 | 14:07:53 | 14:08:53 |
| 15 | 11:50:03 | 12:08:18 | 12:36:41 | 12:45:56 | 12:54:20 | 12:54:35 |
| 16 | 12:24:45 | 12:40:00 | 13:11:45 | 13:21:30 | 13:24:00 | 13:24:15 |
| 17 | 11:37:03 | 11:50:33 | 12:10:09 | 12:18:54 | 12:21:24 | 12:36:02 |
| 18 | 12:41:05 | 12:57:20 | 13:19:53 | 13:31:08 | 12:25:26 | 12:25:56 |

# 4. Pipeline Scripts — Run Order

Scripts must be run in the following sequence. Scripts 01–04 are independent of each other but all depend on the index produced by 00.

| Step | Script | Input | Output | Purpose |
| --- | --- | --- | --- | --- |
| 0 | 00_build_index.py | key.csv | index_10sec.csv | Builds the 10-second time backbone for all participants × phases |
| 1 | empatica_correction.py | raw AVRO files, key.csv | empatica_corrected_10sec.csv + quality HTML | Full NeuroKit2-based physiological processing (preferred Empatica output) |
| 1b | 01_build_empatica.py | raw AVRO files, key.csv | empatica_10sec.csv | Simpler Empatica build (no NeuroKit2); use only if empatica_correction.py fails |
| 2 | 02_build_ucm.py | UCM data.csv files, key.csv | ucm_10sec.csv | UCM Hepia backpack: environment, thermal comfort, air quality, noise |
| 3 | 03_build_atmo_lys.py | Atmo/LYS rawdata CSVs, key.csv | atmo_lys_merged.csv | Atmotube (PM, VOC, AQS) + LYS button (light) for both sensors |
| 4 | 04_build_eyetracker.py | Processed eyetracker CSVs, key.csv | eyetracker_10sec.csv | Pupil Labs Neon: gaze, pupil diameter, blink/fixation/saccade metrics |
| 5 | 05_merge_all.py | All four 10-sec outputs + questionnaires | mergeddata_10sec.csv | Master wide-format file: all sensors + survey scores at 10-sec resolution |

# 5. Script Details

## 5.1  00_build_index.py — Time Backbone

Generates one row per 10-second slot for each participant × phase combination by iterating over key.csv. The result, index_10sec.csv, has columns ParticipantID, PhaseID, Datetime, Date. All subsequent scripts left-join their sensor output onto this backbone, guaranteeing a complete time grid even where sensor data is missing (NaN fill).

Resolution: 10 seconds (floor/ceil applied to phase boundaries)

All times in UTC, no timezone info stored (naive timestamps)

Output: 9,469 rows covering all 17 participants × 5 phases

NOTE: Known bug: column name format mismatch with key.csv causes P4 WalkG and P4 WalkU to appear at wrong UTC times. Pending fix before next re-run.

## 5.2  empatica_correction.py — Physiological Signals (Primary)

This is the preferred Empatica processing script. It reads raw AVRO binary files from the Empatica Embrace Plus wristband and applies NeuroKit2 (nk) for signal cleaning and feature extraction.

### Inputs

Raw AVRO files: raw data/Empatica Data/YYYY-MM-DD/{device}/*.avro (recursive search)

Phase key: key.csv

### Processing steps

AVRO reading: each file decoded segment by segment using avro.datafile.DataFileReader. Signals extracted at native sampling rates: EDA at 4 Hz, BVP at 64 Hz, accelerometer at 32 Hz, temperature at 1 Hz.

EDA cleaning: nk.eda_process(segment, sampling_rate=4) applied per AVRO segment (typically 5–15 min). Extracts EDA_Tonic (SCL) and EDA_Phasic (SCR) components via Butterworth low-pass decomposition.

BVP / Heart Rate: nk.ppg_process(segment, sampling_rate=64) detects PPG peaks per segment. IBI (inter-beat intervals) computed from consecutive peaks; physiological gate 300–2000 ms (30–200 bpm). Heart rate = 60 / mean_IBI; HRV RMSSD = √mean(ΔIBI²) × 1000 in ms, computed per 10-sec window (min 3 IBIs).

Accelerometer: 32 Hz → 1-sec mean → 10-sec median. Vector magnitude = √(acc_x² + acc_y² + acc_z²).

Short internal gaps ≤ 30 sec linearly interpolated; longer gaps left as NaN.

Quality gate: rows where EDA = 0 AND temperature = 0 flagged as device not worn; set to NaN.

Resampling to 10-sec bins: median aggregation (robust to motion artifact spikes).

### Phase windowing

Phase windows built from key.csv UTC times + 2h (Brussels local) to match AVRO timestamps.

Rows outside all five phase windows are discarded.

Output left-joined onto index_10sec.csv to ensure complete time grid.

### Outputs

Per-date intermediate: raw data/Empatica Data/{date}/P{pid}_empatica_corrected_10sec.csv

Combined phase-filtered: outputs/empatica_corrected_10sec.csv

Quality report: outputs/empatica_corrected_quality_report.html (signal plots per participant × phase)

## 5.3  02_build_ucm.py — Environmental Backpack (UCM Hepia)

Reads 1-second GPS and environment data from the UCM Hepia backpack carried by the researcher throughout the experiment. The UCM records temperature, humidity, air quality (PM1/2.5/10, CO2, NO2, O3, SO2, CO, VOC), sound level (dBA), solar radiation, infrared thermal sensors (8 directions), wind, and derived comfort indices (MRT, UTCI, PET, Humidex).

### Inputs

data.csv files: raw data/Participantwise/P{n}/{Phase}/ucm/data.csv (or timestamped subfolder)

Phase key: key.csv

### Processing steps

Column names extracted from 13-line comment header in data.csv (# prefix).

GPS_time column used as datetime (Brussels local time).

Resampled to 10-second bins (mean) using pandas resample.

Phase windows applied from key.csv; PhaseID assigned to each row.

Left-joined onto index_10sec.csv backbone.

### Output: outputs/ucm_10sec.csv

| Variable group | Example columns | Notes |
| --- | --- | --- |
| GPS | gps_lat, gps_lon, gps_alt, gps_speed | Researcher position |
| Air temperature | air_temp, air_rh, air_t_bot/mid/top | Vertical gradient across backpack |
| Infrared (MRT) | ir_back/front/left/right/up/down, ir_spot_* | 8-direction radiant temperature for MRT calculation |
| Air quality | aq_pm010/025/100, aq_co2, aq_no2_m1, aq_o3_m1 | UCM optical particle counter + electrochemical sensors |
| Sound | snd_dba | A-weighted sound pressure level |
| Solar | sun_gh, sun_alt, sun_az | Global horizontal irradiance + sun position |
| Thermal comfort | mrt, utci, pet, humidex | Derived indices computed on-device |
| Wind | wind_aws, wind_awa, wind_tws, wind_twd | Apparent and true wind speed/direction |

NOTE: UCM PM10 shows Green > Urban in this dataset because the optical counter detects coarse biological particles (pollen, organic dust) in La Cambre park. This is physically real but differs from traffic-related PM measured by Atmotube. Do not use UCM and Atmotube PM interchangeably.

## 5.4  03_build_atmo_lys.py — Air Quality & Light (Atmotube + LYS)

Reads four raw CSV files per participant: LYS1 (participant), LYS2 (researcher), atmotube_left (researcher left side), atmotube_right (researcher right side). Files are located in raw data/Atmo & LYS/rawdata/{pid}_{sensor}.csv.

### Timezone handling (critical)

The key.csv times are in UTC. Sensor files record in Brussels local time (CEST = UTC+2). LYS files have a column named "Time UTC" but the values are Brussels local — a mislabelling confirmed by cross-checking against UCM timestamps.

Phase windows are built as key.csv UTC + 2h to match sensor file timestamps.

Sensor timestamps are converted back −2h before joining the UTC index backbone.

### Processing steps

LYS files: Date + "Time UTC" columns parsed; floored to 10-sec bins.

Atmotube files: Date + Time (12-hr AM/PM format) parsed; floored to 10-sec bins. Some later participant files have no separate Time column — timestamp embedded in Date field (24-hr format).

GPS columns (Latitude, Longitude) dropped from Atmotube output.

All four sensors merged on (ParticipantID, PhaseID, Datetime).

atmo_left device failure (P2, P3): atmotube_left showed 100% NaN measurements across all phases. Fix applied: atmo_left columns proxied from atmo_right values; boolean flag column atmo_left_proxied = True set for affected rows.

### Output: outputs/atmo_lys_merged.csv

| Sensor | Columns | Notes |
| --- | --- | --- |
| LYS1 (participant) | lys1__lux, lys1__kelvin, lys1__medi, lys1__rgbR/G/B/IR, lys1__movement | Light exposure on participant |
| LYS2 (researcher) | lys2__lux, lys2__kelvin, lys2__medi, lys2__rgbR/G/B/IR, lys2__movement | Reference / researcher position |
| Atmotube_right | atmotube_right__pm1/2.5/10, voc, aqs, temperature, humidity, pressure | Right side of researcher |
| Atmotube_left | atmotube_left__pm1/2.5/10, voc, aqs, temperature, humidity, pressure + atmo_left_proxied | Left side; P2 & P3 proxied from right |

NOTE: Current completeness: ~93% of slots have data. After pending fixes (index rebuild, bfill), completeness will reach ~99%. Only P3 WalkU remains unrecoverable (sensor battery died before phase).

## 5.5  04_build_eyetracker.py — Gaze & Pupillometry (Pupil Labs Neon)

Reads per-participant processed eyetracker CSV files from raw data/Eyetracker/Processed eyetracker/P{n}.csv. Each file contains ~6-second resolution gaze and pupil metrics.

### Processing steps

PhaseID assigned by comparing timestamps against key.csv phase windows.

Resampled to 10-second means.

Left-joined onto index_10sec.csv.

### Output columns (eyetracker_10sec.csv)

Pupil: pupil_diameter_avg, pupil_change_rate, eyelid_aperture_avg

Blink: in_blink (proportion), blink_duration_s

Fixation: in_fixation, fixation_duration_s, fixation_rate

Saccade: in_saccade, saccade_duration_s, saccade_amplitude, saccade_peak_velocity

Gaze: gaze_velocity, gaze_dispersion, distance_from_center, gaze_centrality

Derived: stress_composite (composite from pupil + fixation features)

## 5.6  05_merge_all.py — Master Merge

Loads all four 10-second sensor outputs and performs a sequential outer-merge on (ParticipantID, PhaseID, Datetime). Questionnaire scores (STAI anxiety, PRS restorativeness, overall comfort, sub-domain comfort ratings) are merged on (ParticipantID, PhaseID) and broadcast to every 10-second row in that phase.

Output: mergeddata_10sec.csv in the project root.

One row = one 10-second slot for one participant in one phase.

NaN in a column means that sensor had no reading at that moment.

NOTE: 05_merge_all.py must be run after all four build scripts have completed. Uses empatica_corrected_10sec.csv (NeuroKit2 version) as the Empatica input.

# 6. Data Quality and Validation

## 6.1  Completeness Check

After running the pipeline, missing data per participant × phase was calculated. Results before and after planned fixes:

| Gap type | Affected | Cause | Fix | Status |
| --- | --- | --- | --- | --- |
| P3 WalkU — 100% NaN | P3, WalkU (163 rows) | Sensor battery died 1 min before phase start | Not fixable | Exclude from Atmo/LYS analysis |
| P4 WalkG & WalkU — 100% NaN | P4, WalkG (121 rows), P4 WalkU (193 rows) | index_10sec.csv built with wrong column names; phase times off by 2h | Fix 00_build_index.py column name format; rebuild index; re-run 03 | Pending |
| P2 & P3 atmo_left — 100% NaN | P2 & P3, all phases | Device hardware failure (no readings across full day) | Proxy atmo_left from atmo_right; flag with atmo_left_proxied = True | Applied in 03_build_atmo_lys.py |
| Start-of-phase boundary gaps (1–7%) | All participants, all phases | Sensor records at 1-min intervals; index has 10-sec slots → 1–6 NaN rows at phase start | bfill(limit=6) within each (ParticipantID, PhaseID) group | Pending |
| P12 WalkG VOC spike (March file) | P12, WalkG | Single erroneous VOC value (25.56 ppm vs normal 0.05–0.3) | Fixed in April shared file; not present in raw pipeline output | Resolved in April CSV; not in pipeline |

Overall completeness: current pipeline ~93% (→ ~99% after index rebuild + bfill).

## 6.2  Cross-Instrument Validation

Selected sensor signals were cross-validated between independent instruments:

| Pair | Level | Pearson r | R² | Interpretation |
| --- | --- | --- | --- | --- |
| UCM Temp vs Atmo Temp | Participant × phase means | 0.71 | 0.50*** | Good agreement — both thermal sensors track same ambient temperature |
| UCM PM2.5 vs Atmo PM2.5 | Participant × phase means | −0.07 | 0.00 ns | No agreement — instruments at different positions; measure different local air pockets |
| UCM PM10 vs Atmo PM10 | Participant × phase means | −0.31 | 0.10** | Negative: UCM PM10 captures biological coarse particles in park; Atmo captures traffic PM |
| UCM Noise dBA vs Atmo PM2.5 | Participant × phase means | 0.65 | 0.43*** | Strong — both mark the same urban environment; indirect cross-validation |
| UCM sun_gh vs LYS1 lux | Row-level (10 sec) | 0.26 | 0.07*** | Moderate — different physical quantities (W/m² vs lux) but positively correlated |

## 6.3  Environmental Hypothesis Verification

Phase means were compared to verify that measured signals reflect expected environmental conditions (Urban = more polluted, noisier, warmer, brighter open sky; Green = quieter, cooler, lower lux under canopy, more restorative). All key signals confirmed expected direction:

| Signal | Urban mean | Green mean | Direction | Expected? | Source |
| --- | --- | --- | --- | --- | --- |
| Noise dBA (UCM) | 68.1 | 60.9 | Urban > Green | Yes | ucm_10sec.csv |
| PM2.5 (Atmotube) | 6.1 | 5.0 | Urban > Green | Yes | atmo_lys_merged.csv |
| VOC (Atmotube) | 0.14 | 0.08 | Urban > Green | Yes | atmo_lys_merged.csv |
| Air Temp (UCM) | 26.8°C | 24.7°C | Urban > Green | Yes (urban heat island) | ucm_10sec.csv |
| CO2 (UCM) | 672 ppm | 654 ppm | Urban > Green | Yes (traffic exhaust) | ucm_10sec.csv |
| LYS1 lux | 9131 | 4342 | Urban > Green | Yes (tree canopy reduces light) | atmo_lys_merged.csv |
| LYS1 kelvin | 5359 K | 4679 K | Urban > Green | Yes (canopy shifts light warmer) | atmo_lys_merged.csv |
| UTCI thermal stress (UCM) | 26.8 | 24.7 | Urban > Green | Yes | ucm_10sec.csv |
| Comfort score (survey) | 4.69/7 | 5.41/7 | Green > Urban | Yes | ExperimentData_20Apr.csv |
| PRS restorativeness | 2.50/5 | 3.56/5 | Green > Urban | Yes (ART) | ExperimentData_20Apr.csv |
| Anxiety (STAI-6) | 31.4/100 | 25.1/100 | Urban > Green | Yes | ExperimentData_20Apr.csv |
| Tram CO2 | 834 ppm | — | Tram >> outdoor | Yes (enclosed vehicle) | ucm_10sec.csv |

# 7. Output Files Summary

| File | Script | Rows | Columns | Description |
| --- | --- | --- | --- | --- |
| index_10sec.csv | 00_build_index.py | 9,469 | 4 | Time backbone: ParticipantID, PhaseID, Datetime, Date |
| empatica_corrected_10sec.csv | empatica_correction.py | 9,469 | 14 | EDA, EDA_Tonic, EDA_Phasic, BVP, Heart Rate, HRV RMSSD, Accel, Temp (NeuroKit2 processed) |
| ucm_10sec.csv | 02_build_ucm.py | 9,469 | 57 | GPS, air temp, RH, infrared temps (8 dirs), PM1/2.5/10, CO2, NO2, O3, SO2, CO, dBA, solar, wind, MRT, UTCI, PET |
| atmo_lys_merged.csv | 03_build_atmo_lys.py | 9,469 | 43 | LYS1+LYS2 (lux, kelvin, medi, RGB), Atmo_left+right (PM1/2.5/10, VOC, AQS, temp, humidity, pressure) |
| eyetracker_10sec.csv | 04_build_eyetracker.py | 9,469 | 20 | Pupil diameter, blink/fixation/saccade metrics, gaze dispersion, stress composite |
| mergeddata_10sec.csv | 05_merge_all.py | 9,469 | ~130 | All 4 sensor streams + questionnaire scores (STAI, PRS, Comfort) in one wide file |

# 8. Reference Dataset — ExperimentData_20Apr.csv

An independently compiled reference dataset is available at: Field experiment raw data/Complete Participantwise data/Data shared emails/ExperimentData_20Apr.csv. This file was produced by a separate processing workflow and contains 1-minute resolution data with all sensors merged including UCM, Empatica, Eyetracker, surveys, and demographic data (2,323 rows × 263 columns). It serves as the validation baseline.

Completeness: LYS, UCM, Empatica 100%; Atmo_right PM 100%; Atmo_left ~88% (P2/P3 device failure); eyetracker pupil ~81%.

The April version corrected a P12 WalkG VOC spike (25.56 ppm → 0.076 ppm) present in the March version.

For cross-phase analysis requiring all sensors including Empatica and surveys, use this file directly.

For sub-minute time-series or 10-second resolution analysis, use the pipeline outputs.

NOTE: The pipeline (atmo_lys_merged.csv) and reference dataset (ExperimentData_20Apr.csv) show identical rank ordering of phase means for LYS lux and Atmotube temperature, confirming correct timezone handling and phase assignment in the pipeline.

# 9. Known Issues and Pending Fixes

| Issue | Script | Impact | Fix required |
| --- | --- | --- | --- |
| 00_build_index.py uses wrong column format Start_time_{ph} / End_time_{ph} (key.csv has {ph}_start / {ph}_end) | 00_build_index.py | P4 WalkG (121 rows) and P4 WalkU (193 rows) appear at wrong UTC times in index → 100% NaN in all sensor outputs for these phases | Rename s_col = f"{ph}_start", e_col = f"{ph}_end"; rebuild index; re-run scripts 01–05 |
| bfill not applied to start-of-phase boundary gaps | 03_build_atmo_lys.py | 1–7% NaN at start of each phase across all participants (sensor records at 1-min; index at 10-sec) | Add bfill(limit=6) within each (ParticipantID, PhaseID) group after index join in 03; apply analogously in 01, 02, 04 |
| 04_build_eyetracker.py references Key_for_data_splitting_updated_05May.csv, not key.csv | 04_build_eyetracker.py | Uses old key file; may produce different phase windows than other scripts | Update KEY_FILE path to key.csv |

# 10. Running the Pipeline

All scripts require Python 3.9+ with the following packages:

pandas, numpy, scipy — core data processing

avro-python3 (or avro) — Empatica AVRO binary reading

neurokit2 — NeuroKit2 EDA/BVP signal processing (empatica_correction.py only)

matplotlib — quality report plots

python-docx — documentation generation (temp scripts only)

Run in order from the project root:

python scripts\00_build_index.py

python scripts\empatica_correction.py

python scripts\02_build_ucm.py

python scripts\03_build_atmo_lys.py

python scripts\04_build_eyetracker.py

python scripts\05_merge_all.py

NOTE: Raw data files must be in place before running. Scripts are read-only on raw data; all outputs go to outputs/. Running a script twice will overwrite its output file.

# Appendix A — Sensor Description

| Sensor | Device | Worn by | Variables measured | Sampling rate |
| --- | --- | --- | --- | --- |
| Empatica | Empatica Embrace Plus | Participant (wrist) | EDA (SCL/SCR), skin temperature, BVP, heart rate, HRV RMSSD, accelerometer, gyroscope | 4–64 Hz native → 10-sec bins |
| LYS1 | LYS button | Participant (lapel/clothing) | Illuminance (lux), colour temperature (kelvin), melanopic EDI, RGB+IR channels, movement | Variable → 10-sec bins |
| LYS2 | LYS button | Researcher (lapel) | Same as LYS1 — reference / researcher position | Variable → 10-sec bins |
| Atmotube left | Atmotube Pro | Researcher (left side) | PM1, PM2.5, PM10, VOC, Air Quality Score, temperature, humidity, pressure | ~1 min → 10-sec bins |
| Atmotube right | Atmotube Pro | Researcher (right side) | PM1, PM2.5, PM10, VOC, Air Quality Score, temperature, humidity, pressure | ~1 min → 10-sec bins |
| UCM backpack | UCM Hepia Environmental Backpack | Researcher (back) | GPS, air temp & RH (3 heights), 8-dir infrared, PM1/2.5/10, CO2, NO2, O3, SO2, CO, sound dBA, solar GHI, wind, MRT, UTCI, PET, Humidex | 1 Hz → 10-sec bins |
| Eye tracker | Pupil Labs Neon | Participant (glasses) | Gaze (x,y), pupil diameter (L+R+avg), blink/fixation/saccade events, gaze velocity, gaze dispersion | ~200 Hz native → 6-sec processed → 10-sec bins |

# Appendix B — Experiment Phases

| Phase ID | Description | Environment | Mode | Location |
| --- | --- | --- | --- | --- |
| BikeU | Urban cycling | Urban | Bike | Usquare / Etterbeek → Montgomery, Brussels |
| WalkU | Urban walking | Urban | Walk | Usquare / Etterbeek → Montgomery, Brussels |
| BikeG | Green cycling | Green | Bike | La Cambre park, Brussels |
| WalkG | Green walking | Green | Walk | La Cambre park, Brussels |
| Tram | Tram (seated transit) | Tram | Sit | Boondael Gare → Etterbeek Gare (Tram 8) |