Short-Term Physiological Response to Dynamic Environmental Exposure

Data Processing Methodology  |  Field Experiment, Brussels 2025

N = 17 participants (P2–P18)  |  Generated 06 May 2026

1. Study Overview

Seventeen participants (P2–P18) each completed one day of data collection in Brussels, cycling, walking, and riding the tram across urban and green environments. Physiological and environmental data were recorded continuously and binned to 10-second windows for analysis. All timestamps are aligned to Europe/Brussels (CEST, UTC+2).

| Phase | Environment | Mode | Route |
| --- | --- | --- | --- |
| Bike_U | Urban | Cycling | Usquare -> Etterbeek -> Montgomery |
| Walk_U | Urban | Walking | Usquare -> Etterbeek |
| Bike_G | Green | Cycling | La Cambre park |
| Walk_G | Green | Walking | La Cambre park |
| Tram | Transit | Sitting | Boondael Gare -> Etterbeek Gare |

| Sensor | Script | Output file |
| --- | --- | --- |
| Empatica Embrace Plus (wrist) | 01_build_empatica.py | empatica_corrected_10s.csv |
| Pupil Labs Neon (eye tracker) | 04_build_eyetracker.py | eyetracker_10sec.csv |
| UCM Hepia backpack | 02_build_ucm.py | ucm_10sec.csv |
| Atmotube Left & Right | 03_build_atmo_lys.py | atmo_lys_merged.csv |
| LYS Button 1 & 2 | 03_build_atmo_lys.py | atmo_lys_merged.csv |
| All sensors merged | 05_merge_all.py | mergeddata_10sec.csv |

2. Empatica Embrace Plus  (01_build_empatica.py)

The Empatica Embrace Plus is a wrist-worn device that records EDA, BVP (photoplethysmography), skin temperature, and 3-axis accelerometer data. Raw data are stored as AVRO files using Empatica's v6 schema (Empatica, 2023). Signal processing uses NeuroKit2 v0.2.9 (Makowski et al., 2021). Each AVRO file is processed independently to prevent NeuroKit2 from treating inter-file gaps as signal loss.

Processing steps

Load AVRO files. Each file is decoded using the Empatica v6 schema. Accelerometer raw integer values are converted to physical units (g) using the imuParams.scaleFactor embedded in each file.

EDA (4 Hz). nk.eda_process(sr=4) is called on each AVRO segment. This applies cvxEDA (Greco et al., 2016), which decomposes the raw EDA signal into a tonic component (skin conductance level, SCL -- slow-moving baseline) and a phasic component (skin conductance response, SCR -- event-locked spikes). Segments shorter than 20 seconds are skipped for decomposition; raw EDA is retained.

BVP / Heart Rate (64 Hz). nk.ppg_process(sr=64) detects systolic peaks. A template signal quality index (SQI) is computed as the Pearson correlation between each beat waveform and the session mean template (Elgendi et al., 2016). Windows with SQI < 0.5 are marked NaN. Heart rate is gated to 30--220 bpm. Reconnection AVRO buffers (~9 s, filled with 0.0 by the device) are identified and handled with a NaN-guard before processing.

HRV -- RMSSD (derived from BVP). Inter-beat intervals (IBI) are extracted from detected peaks and gated to 0.30--2.00 s (Task Force, 1996). Artifact correction uses the Lipponen & Tarvainen (2019) algorithm. RMSSD is computed per 10-second window requiring at least 3 clean peaks. For statistical analysis, lnRMSSD (natural log) is used. Ultra-short RMSSD validity: r = 0.97 vs 5-minute window (Esco & Flatt, 2014).

Skin Temperature (1 Hz). 10-second median per bin. No further processing required.

Accelerometer (3-axis, ~64 Hz). Vector magnitude |g| = sqrt(ax^2 + ay^2 + az^2). 10-second median. Used as activity covariate in all LME models to separate physiological arousal from motion artefact.

Phase labelling and 10-second binning. Timestamps are matched to participant phase windows from key.csv (UTC, shifted to local Brussels time). All signals are aggregated to 10-second bins. Output: outputs/empatica_corrected_10s.csv

Data quality (PHANTOM framework -- Bent et al., 2020)

Coverage thresholds: EDA GOOD > 70%, FAIR 40--70%; HR GOOD > 60%, FAIR 30--60%. P5, P6, P12, and P16 are excluded from all HR/HRV analyses due to device connection failures or total PPG signal loss. EDA is retained where coverage exceeds 40%.

| PID | EDA % | HR % | Phases | Status |
| --- | --- | --- | --- | --- |
| P2 | 99.6 | 84.6 | 4/5 | GOOD |
| P3 | 97.8 | 75.0 | 3/5 | FAIR |
| P4 | 96.3 | 64.7 | 4/5 | FAIR |
| P5 | 61.4 | 13.4 | 5/5 | POOR -- 251/268 AVROs are 9-s reconnect buffers; excl. HR/HRV |
| P6 | 68.2 | 32.1 | 5/5 | POOR -- 148 AVROs Bluetooth loss; excl. HR/HRV |
| P7 | 99.1 | 61.1 | 3/5 | FAIR |
| P8 | 98.0 | 80.2 | 3/5 | FAIR |
| P9 | 99.0 | 79.9 | 2/5 | FAIR |
| P10 | 96.0 | 70.5 | 4/5 | GOOD |
| P11 | 88.1 | 42.3 | 2/5 | FAIR |
| P12 | 64.5 | 39.3 | 2/5 | POOR -- 161 AVROs Bluetooth loss; excl. HR/HRV |
| P13 | 98.3 | 71.3 | 3/5 | FAIR |
| P14 | 88.5 | 50.2 | 5/5 | GOOD |
| P15 | 80.8 | 40.9 | 4/5 | GOOD |
| P16 | 88.4 | 32.2 | 5/5 | POOR -- total PPG loss; excl. ALL HR/HRV |
| P17 | 90.6 | 45.4 | 4/5 | GOOD |
| P18 | 93.5 | 64.3 | 3/5 | FAIR |

3. Eye Tracker -- Pupil Labs Neon  (04_build_eyetracker.py)

The Pupil Labs Neon smart glasses record binocular gaze, fixations, saccades, blinks, pupil diameter, eyelid aperture, IMU, and synchronised audio at approximately 25 Hz. Recordings were exported to CSV from Neon Player. Some sessions had corrupted MP4 files (missing moov atom from app crashes); these were recovered using the pl-recover-recording tool (Pupil Labs, open-source) before CSV export.

Processing steps

Load raw CSVs per session and phase: gaze_positions.csv, fixations.csv, saccades.csv, blinks.csv, imu.csv, 3d_eye_states.csv. Gaze is used as the base time-series.

Timestamp conversion. All timestamps are converted from nanoseconds to seconds. A relative_time column is derived from the recording start.

Event labelling. For each fixation, saccade, and blink event, timestamps are matched to gaze rows using inclusive range masks. Boolean flags (in_fixation, in_saccade, in_blink) and associated metrics (duration, amplitude, peak velocity) are added to each gaze row.

IMU and 3D eye state merge. IMU data (gyro x/y/z, accelerometer x/y/z, roll/pitch/yaw) and 3D eye state data (pupil diameter left/right, eyelid aperture left/right) are merged onto the gaze time-series using pd.merge_asof with direction=nearest.

Audio feature extraction. The session audio file is loaded with librosa. For each 10-second window: RMS energy, zero-crossing rate, spectral centroid, spectral bandwidth, MFCCs (mean of 13 coefficients), tempo, and speech probability are computed. An audio_stress_indicator is derived via min-max normalisation.

10-second aggregation. All gaze-level data are floored to 10-second bins. Numeric metrics are averaged; boolean event flags by majority vote (> 50%). Fixation/saccade/blink counts and rates are computed per bin.

Stress composite score. fixation_count_scaled and fixation_duration_scaled (inverted) are min-max normalised. Their mean forms stress_composite (0--1). A categorical stress_level (Low / Moderate / High / Very High) is derived by quartile binning.

Phase labelling. Each bin is tagged with PhaseID from the participant key.csv. Output: outputs/eyetracker_10sec.csv

Key output columns

Pupil & eyelid: pupil_diameter_avg, pupil_change_rate, eyelid_aperture_avg -- sympathetic arousal / cognitive load proxy (Kahneman & Beatty, 1966). NOTE: confounded by ambient luminance (Loewenfeld, 1999); always include LYS/UCM lux as covariate.

Fixation: in_fixation, fixation_duration_s, fixation_rate -- attentive scanning (Urban) vs deep processing (Green)

Saccade: saccade_amplitude [deg], saccade_peak_velocity [deg/s] -- scanning breadth and cortical arousal marker

Blink: in_blink, blink_duration_s -- cognitive fatigue indicator

Gaze quality: gaze_velocity, gaze_dispersion, gaze_centrality

Audio: audio_rms, audio_stress_indicator, audio_stress_level

Composite: stress_composite, stress_level

4. Environmental Sensors

4.1  UCM Hepia Backpack  (02_build_ucm.py)

The UCM Hepia backpack is a multi-sensor environmental logger carried by the researcher alongside the participant. It records temperature, relative humidity, sound level (LAeq), illuminance, and GPS position at 1 Hz, providing ground-truth micro-environmental conditions along the route.

Load raw CSV files from raw data/ucm/ (fallback: E:/Complete Participantwise data/Ucm_03May). Files contain: timestamp, T_ext (deg C), RH (%), LAeq (dB(A)), lux, lat/lon, CO2, PM, AQI.

Timestamp parsing and timezone alignment to Brussels local (CEST).

Phase labelling using participant key.csv phase windows.

10-second resampling: mean per bin for all environmental signals.

GPS: lat/lon retained per bin for spatial segment labelling (~50 m bins) and route heat maps.

Output: outputs/ucm_10sec.csv

4.2  Atmotube Pro -- Left & Right  (03_build_atmo_lys.py)

Two Atmotube Pro units were mounted on the left and right shoulders of the researcher's backpack. The directional placement provides wind-direction-aware air quality measurement: mean(left, right) gives overall PM dose; |left - right| indicates directional asymmetry (proxy for traffic proximity direction).

Load CSVs for atmotube_left and atmotube_right from participant folders.

Timestamp alignment to Brussels local time.

Phase labelling from key.csv.

10-second resampling: mean PM1, PM2.5, PM10, VOC, CO, AQS per bin.

Directional asymmetry column: abs(PM2.5_left - PM2.5_right) added.

Output appended to outputs/atmo_lys_merged.csv

4.3  LYS Button 1 & 2  (03_build_atmo_lys.py)

Two LYS light sensors were worn continuously: LYS1 on the participant (chest) and LYS2 on the researcher. Each records illuminance (lux), correlated colour temperature (CCT), and melanopic equivalent daylight illuminance (mEDI) at 1 Hz.

Load LYS1 and LYS2 CSVs. NOTE: the column header reads "Time UTC" but the values are Brussels local time (CEST). Phase windows from key.csv (UTC) are shifted +2 hours before matching to compensate for this mislabelling.

Phase labelling and 10-second mean aggregation.

Columns prefixed lys1_ and lys2_ in the output.

Primary use: (1) luminance covariate to deconfound pupil dilation comparisons between Urban and Green phases; (2) circadian light exposure descriptor.

Output appended to outputs/atmo_lys_merged.csv

References

Makowski, D. et al. (2021)  NeuroKit2: A Python Toolbox for Neurophysiological Signal Processing. Behavior Research Methods, 53, 1689--1696.

Greco, A. et al. (2016)  cvxEDA: A Convex Optimization Approach to Electrodermal Activity Processing. IEEE Transactions on Biomedical Engineering, 63(4), 797--804.

Elgendi, M. et al. (2016)  Systolic Peak Detection in Acceleration Photoplethysmograms Measured from Emergency Responders. PLOS ONE, 8(4), e76585.

Lipponen, J. A. & Tarvainen, M. P. (2019)  A robust algorithm for heart rate variability time series artefact correction using novel beat classification. Journal of Medical Engineering & Technology, 43(3), 173--181.

Task Force of the European Society of Cardiology (1996)  Heart rate variability: Standards of measurement, physiological interpretation, and clinical use. Circulation, 93(5), 1043--1065.

Esco, M. R. & Flatt, A. A. (2014)  Ultra-Short-Term Heart Rate Variability Indexes at Rest and Post-Exercise in Athletes. Journal of Sports Science & Medicine, 13, 535--541.

Bent, B. et al. (2020)  Investigating sources of inaccuracy in wearable optical heart rate sensors. npj Digital Medicine, 3, 18.

Kahneman, D. & Beatty, J. (1966)  Pupil diameter and load on memory. Science, 154(3756), 1583--1585.

Loewenfeld, I. E. (1999)  The Pupil: Anatomy, Physiology, and Clinical Applications. Butterworth-Heinemann.

Empatica (2023)  Embrace Plus Developer Documentation -- AVRO data format specification v6.